“How to use” guide for the River Restoration Centre’s monitoring Protocol:

Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use rrc update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Model, survey and prioritise areas of diffuse pollution across the rural Ribble catchment | Understanding of diffuse and pin point catchment sources of pollution. | Review as part of end of project assessment | At end of the project | RRT staff | n/a | In kind (staff time) | N/a | Priority: Medium | Y | Evaluated as part of final project assessment |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Deliver pinpoint farm visits | Increased farm engagement | Record number of farm projects delivered as result of farm visits | Record periodically, assess at end of project | RRT staff | N/a | In kind (staff time) | N/a | Priority: Medium | Y | As above |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Deliver works (via farm visits) to reduce diffuse pollution in the rural Ribble catchment | Has diffuse pollution reduced at the catchment scale? | Fixed Pont Photography to be used to assess extent of sediment input It had been hoped that we could do water quality sampling. We have some data, but limited. | For FPP pre-works, during-works, immediately post and some time post-works. | RRT staff | None | In kind (staff time) | WFD failure for pH, ammonia, phosphate Also related to failure for fish, invertebrates | Priority: High | Y | As above |
|  |  |  |  |  |  |  |  | Confidence: Medium (Ideally would be directly measuring specific pollutant failures via pH, ammonia, phosphate). Could be high dependent on specific type of farm works being delivered- this would be farm/site specific. |  |  |
|  |  | Invertebrate sampling at sites across the catchment. | Using the Rivefly partnership method developed by EA, SEPA and FBA. 45 sites on monthly or bimonthly basis. |  |  |  |  | Priority: High |  |  |
|  |  |  |  |  |  |  |  | Confidence: Medium (ideally monitoring would continue beyond project timeline) |  |  |
|  |  | Electrofishing at PIT tagging undertaken for fish passage elements of this project. | As part of network of 300 sites across the Ribble catchment. Sites selected with EA staff. Annually |  | We have 4 years electrofishing data for the 300 sites across catchment. |  |  | Priority: High (for fish passage elements) |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |