Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

National Trust – Peatland Project  HUM017

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD objective is this helping to achieve? If not WFD specify other e.g. CaBA | Priority High/medium/low linked to WFD or other designation Confidence High/medium/low robustness of monitoring | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use rrc update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package 1 Block all remaining gully systems on the Edge | POC | TIMS time integrated mass flux samplers | Autumn; TIMS are installed and left to collect sediment for 4 weeks. This provides a snapshot of sediment flux from blocked / unblocked gullies. | MFF | Data from a pilot study carried out at the package 1 site in early Autumn 2013. | TBC - January | Ecology moderate | Priority: H | Yes | Comparison of POC flux from blocked / unblocked gullies. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Water quality | Fortnightly spot sampling | Fortnightly throughout the project. | MFF | Data from Bamford Heavy Metals Project provides 1 year of data for 8 sites. | £9550 |  | Priority: H | Yes | Look at change over time, 2013 data compared with 2014 data. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Sediment accumulation behind gully blocks | Measure depth of sediment on the upstream side of the dams constructed across the erosion gullies. | Immediately after gully blocking and 1 year after gully blocking. | MFF | None | £814 |  | Priority: H | Yes | Monitor sediment accumulation behind dams across erosion gullies |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued) |  |  |
|  | Runoff | Flow | Semi-permanent gauging sites with rated sections and water levels recorded in stilling wells | MFF | Existing flow data collected as part of ‘Making Space for Water’ project from Severn Trent Water and Manchester University. | £4560 |  | Priority: H | Yes | Long term flow monitoring. Data will also be used to calculate flux of carbon & heavy metals. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Surface topography | Standard LiDAR survey | Will be surveyed this year (2013) and again in 2014 | BlueSky (GIS Specialist) | 2004 survey data | £6380 |  | Priority: H |  | Analysis of peat accumulation, erosion and redisposition, particular within gullies. Will compare surveys from 2004, 2013 and 2014 |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Landscape changes | Fixed Point photography | During baseline survey and 1 year later during repeat survey. | MFF | none | Included in cost of sediment accumulation survey. |  | Priority: H | Yes | Comparisons of changes in landscape features before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 1 Restrict bare peat to less than 10% | Percentage of covered peat | Aerial photographs | June 2013 and June 2014. | BlueSky GIS specialist | Previous aerial photographs | Included in standard LiDAR survey | Ecology moderate | Priority: H | Yes | Comparison of percentage bare peat before and after works carried out |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | LiDAR | Will be surveyed this year (2013) and again in 2014 | consultant | 2004 survey data | Included in standard LiDAR survey |  | Priority: H | Yes | Analysis of changes in surface level and coverage of peat . Will compare surveys from 2004, 2013 and 2014 |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 1 Create sphagnum colonies on 80% of suitable habitat by July 2015 | Percentage of cover of sphagnum moss | Fixed quadrats and fixed point photography | Immediately after Sphagnum application and July 2015. | MFF | Baseline transect survey carried out summer 2013 | £1760 | Ecology moderate | Priority: H | Yes | Compare percentage of cover of sphagnum moss before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 2 Block all remaining gully by July 2015 | POC | TIMS time integrated mass flux samplers | Autumn; TIMS are installed and left to collect sediment for 4 weeks. This provides a snapshot of sediment flux from blocked / unblocked gullies. | MFF | Data from a pilot study carried out at the package 1 site in early Autumn 2013. | TBC - January | Ecology moderate | Priority: H | Yes | Comparison of POC flux from blocked / unblocked gullies. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Water quality | Fortnightly spot sampling | Fortnightly throughout the project. | MFF | Data from Bamford Heavy Metals Project provides 1 year of data for 8 sites. | £20010 |  | Priority: H | Yes | Look at change over time, 2013 data compared with 2014 data. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Sediment accumulation behind gully blocks | Measure depth of sediment on the upstream side of the dams constructed across the erosion gullies. | Immediately after gully blocking and 1 year after gully blocking. | MFF | None | £1702 |  | Priority: H | Yes | Monitor sediment accumulation behind dams across erosion gullies |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued) |  |  |
|  | Runoff | Flow | Semi-permanent gauging sites with rated sections and water levels recorded in stilling wells | MFF | Existing flow data collected as part of ‘Making Space for Water’ project from Severn Trent Water and Manchester University. | £9535 | Ecology moderate | Priority: H | Yes | Long term flow monitoring. Data will also be used to calculate flux of carbon & heavy metals. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Surface topography | Standard LiDAR survey | Will be surveyed this year (2013) and again in 2014 | BlueSky (GIS Specialist) | 2004 survey data | £13340 |  | Priority: H | Yes | Analysis of peat accumulation, erosion and redisposition, particular within gullies. Will compare surveys from 2004, 2013 and 2014 |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Landscape changes | Fixed Point photography | During baseline survey and 1 year later during repeat survey. | MFF | none | Included in cost of sediment accumulation survey. |  | Priority: H | Yes | Comparisons of changes in landscape features before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 2 create sphagnum colonies on 80% 0f suitable habitat by July 2015 | Percentage of cover of sphagnum moss | Fixed quadrats and fixed point photography | Immediately after Sphagnum application and July 2015. | MFF | Baseline transect survey carried out summer 2013 | £3680 | Ecology moderate | Priority: H | Yes | Compare percentage of cover of sphagnum moss before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 2 Increase in cotton grass and moorland species | Percentage cover of cotton grass and moorland species | Fixed point photography | Immediately after planting and 1 year later | MFF | none | £2775 | Ecology moderate | Priority: H |  | Compare percentage of cover of cotton grass and moorland species before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | quadrats | Immediately after planting and 1 year later | MFF | none | Included in above cost |  | Priority: H |  | Statistical analysis of coverage of cotton grass and moorland species to look at survival and establishment of these species |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 3 Restrict bare peat to less than 25% by re-seeding | Percentage of covered peat | Aerial photographs | June 2013 and June 2014. | BlueSky GIS specialist | Previous aerial photographs | Included in standard LiDAR survey | Ecology moderate | Priority: H | Yes | Comparison of percentage bare peat before and after works carried out |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Quadrats | Before works and 1 year later. | MFF | none | £4625 |  | Priority: H |  | Measure of how much cover there is and floral assemblage diversity |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Package 3 Install 500 dams on Kinder plateau by July 2015 | Water Quality | Fortnightly spot sampling | Fortnightly throughout the project. | MFF | Data from Bamford Heavy Metals Project provides 1 year of data for 8 sites. | £13920 |  | Priority: H | Yes | Look at change over time, 2013 data compared with 2014 data. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | POC | TIMS time integrated mass flux samplers | Autumn; TIMS are installed and left to collect sediment for 4 weeks. This provides a snapshot of sediment flux from blocked / unblocked gullies. | MFF | Data from a pilot study carried out at the package 1 site in early Autumn 2013. | TBC - January |  | Priority: H | Yes | Comparison of POC flux from blocked / unblocked gullies. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Runoff | Flow | Semi-permanent gauging sites with rated sections and water levels recorded in stilling wells | MFF | Existing flow data collected as part of ‘Making Space for Water’ project from Severn Trent Water and Manchester University. | £6633 |  | Priority: H | Yes | Long term flow monitoring. Data will also be used to calculate flux of carbon & heavy metals. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Water table height | Dipwells | Automatic logging of groundwater levels plus 14 manual dip wells over Autumn (Sep to Nov) | MFF | none | £25000 |  | Priority: H |  | Measurement of effect of dams in raising groundwater levels in the peat (ie more water being retained) |
|  |  |  |  |  |  |  |  | Confidence M (will need to delineate high GW levels due to wet winter from increased GW levels due to works |  |  |

Nearly all of the work / monitoring is within the Ashop Catchment. The only monitoring being carried out in the Alport is plug plant monitoring at Miry Clough which falls within work package 2. No works / monitoring is being carried out at Ladybower Lake.

Reporting Note: Interim reports will be produced in years 1 and 2 with a final report in year 3