“How to use” guide for the River Restoration Centre’s monitoring Protocol:

Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – (for methodology, refer to RRC’s Practical river monitoring guidance 2011) X number of photos (state if known) & if points indicated on map (Y/N) | E.g. before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: High (All CRF projects were encouraged to prioritise FPP) | Yes/ No | A time-series of fixed point photographs State if included in e.g. final report |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if both confidence and priority are High) |  |  |

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Improvements to Forestry Management practice | Reduced contamination from forest roads; road surface and any fuel through advising the forestry commission and a private forestry business on road drain re-directions. | Site visits and photography to identify drain locations and connectivity to river system. Project Officer visits in correct conditions and advises and then visits again. | During and in weeks after road drainage works. Also after high flows in the months following works. Measurements as part of Greenlee Lough monitoring will also pick up peaks in sediment from parts of the forest. | TRT | Walkover surveys of all streams running through forests to identify potential issues caused by forestry. Including photography and turbidity readings. | In-kind | Overall physico-chemical status: high Morphology: good | Priority: Medium (not fixed-point) | Yes | Comparison of photos taken during walkover surveys, before and after works and after heavy rainfall. |
|  |  |  |  |  |  |  |  | Confidence: Medium (visual monitoring possibly subjective) |  |  |
|  |  | Turbidity measurements - YSI electrochemical instrumentation. | EA have sondes in inflow streams to measure turbidity, ammonia and conductivity. Samples for phosphate are collected each month and sent to the lab for analysis. | EA |  | EA not charging for their contribution. |  | Priority: High (not to deteriorate) |  | Comparison of sonde data. Data documented on spread sheet. Probably included in final report. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Improvements to Forestry Management practice | Low level of sediment during felling near streams by installing leaky dam structures. However, these can only be put in place after felling for H&S. Once in place at down-stream sites, they will help after subsequent felling at upstream sites. Also working with foresters. | Turbidity readings - YSI hand held electro-chemical sonde. EA measurements further downstream | During and in weeks after felling. Also after high flows in months following felling. Hope to use leaky dams to push water and sediment onto flood-plain therefore also taking pH peak out. Greenlee Lough monitoring will also pick up peaks in sediment from parts of the forest. | TRT | Walkover surveys of all streams running through forests to identify potential issues caused by forestry. Including photography and turbidity/pH readings. | In-kind | Morphology: good | Priority: High (not to deteriorate) | Yes, but difficult to co-ordinate with forestry management schedules. | Comparison with data and photographs taken during walkover surveys. Comparison of sonde data. Data documented on spread sheet. Probably included in final report. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Photography | During and in weeks after felling. Also after high flows in the months following felling. | TRT |  |  |  | Priority: Medium (not fixed-point) |  |  |
|  |  |  |  |  |  |  |  | Confidence: Medium |  |  |
| Improvements to Greenlee Lough water quality | Reduced phosphate and sediment reaching lough by cleaning dirty water separation and wetland creation at problem farm. Also review of cattle access sites. | Electrochemical sondes at inlets and outlets of lough. Flow measurements and samples for lab analysis (phosphate and sediment load). | Sondes in situ monitoring constantly. Physical samples taken ~quarterly. Ideally needs to be at the same time of year or after similar rainfall events. | EA | EA survey of lough itself | EA not charging for their contribution. | SSSI: Protection against siltation and eutrophication | Priority: High (for SSSI designation) | Yes | Comparison of turbidity and phosphate before and after works completed. Data documented on spread sheet. Probably included in final report. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Improvements in water quality of Haltwhistle Burn. | Increased water quality and variation in habitats by green engineered structures to protect river bank collapse and therefore contaminated soils reaching burn. | Continuous monitoring using electrochemical sonde: DO, pH, ammonium, temperature and turbidity. | Continuous until EA withdraw sondes. | EA | None | EA | Overall physico-chemical status: high | Priority: High (not to deteriorate) | Yes (might withdraw their sonds before end of CRF) | Comparison of results before and after works, around events such as storms and changes to farming practice. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Fixed-point photography | Before, during and after works (3 sets). Monitoring to be taken on by community. | TRT | None | In-kind | Morphology: good | Priority: High | Yes | Comparison of photos taken before and after works. Recorded |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Reduced erosive force of flooding on farm land | Evidence of flow being slowed and sediment depositing on floodplain by installing timber sluices and leaky dams. | Flow - Valeport flow meter. Fixed divers at some locations. | Before and after construction of sluices/dams. During and shortly after periods of high flow. | TRT / farmer / volunteer | Photographs from farmers and residents can be digitally assessed for volume and sediment. | PhD student - Newcastle University. | Hydrology: high Morphology: good | Priority: Medium (morphology already defined as good) | Yes | Comparison of flow data. Thesis Comparison of photographs during flood events before and after works completed. |
|  |  |  |  |  |  |  |  | Confidence: High (if baseline data is sufficient) |  |  |
|  |  | Fixed-point photography | Before, during and after (to be done when location decided) |  |  |  |  | Priority: High |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Creation of riparian buffer strip | Reduced input from cattle (poaching, voiding), improved vegetative riparian cover and bank structure | WQ sampling: phosphate, ammonia, turbidity, conductivity. | Year round, although vegetation best observed in summer. Before and after cattle grazing at given site. | TRT / volunteer | None | Lab charge, in-kind | Phosphate: high Ammonia: high Morphology: good | Priority: Medium (WFD specific quality elements defined as high) | Yes | Comparison of WQ data. Documented in spread sheet, probably included in final report. Comparison of photographs before and after works completed. Recorded |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Fixed-point photography | Before, during, after (to be done) |  |  |  |  | Priority: High |  |  |
|  |  |  |  |  |  |  |  | Confidence: High (for riparian cover; Medium - nutrients) |  |  |
| Improved tree management | Evidence of bank stabilisation by planting at landslide site. Also removal of tall trees to remove sail effect. | Fixed-point photography | Before, during and after works | TRT | Some historic observations and photos | In-kind / volunteer | Morphology: good | Priority: High | Yes | Comparison of photographs. Rerecorded |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Enhanced fish passage | Fish upstream of impassable section. Three sites: addressing gravel movements at downstream confluence (bank management); baffle mitigation at two upstream sites (one culvert). | Electro-fishing - species count, size and age structure. | Post-monitoring: Electro-fishing permitted season July – Sept (2014). Looking for funding to extend monitoring. | TRT | Electro-fishing data from summer 2010-2013. | In-kind / volunteer | Fish: poor | Priority: High | Yes | Comparison of results from previous surveys versus after fish easement construction. Documented in spread sheet, probably included in final report. |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit; High if post-monitoring is extended) |  |  |
| To assess whether flood peaks have been reduced due to works | Slower rise and lower flood peaks by managing gauge boards, leaky dams, buffer strips, timber sluices softening banks in gorge, off and on line ponds. River levels to be coordinated with other observations (divers, historic data). | Volunteer to read gauge boards installed by CRF project. | Before, during and after storm events. Regularly throughout year. | Volunteers | None (except some historic or anecdotal) | Volunteer | Hydrology: high Morphology: good | Priority: High | Yes | Recorded data compared of before, during and after storm events/works. |
|  |  |  |  |  |  |  |  | Confidence: Medium (little baseline data) |  |  |
|  |  | Divers to be installed at a few key locations. | Before, during and after storm events. Regularly throughout year. | Newcastle University – to be passed to volunteer | None | Gauge boards, PhD Newcastle/ volunteer | Hydrology: high Morphology: good | Priority: Medium | Yes - gage boards in place | Comparison of data before, during and after storm events and works. Recorded and included in thesis, probably in final report. |
|  |  |  |  |  |  |  |  | Confidence: Low (unless baseline data exists to compare with) |  |  |
| Effect of local rainfall pattern | Catchment response to rainfall accumulation. | Rain gauge at 2 different locations in catchment. | Daily, ongoing | Newcastle University / volunteers | Met office data which is not always local enough. | Equipment, volunteer | Hydrology: high | Priority: Medium (hydrology already defined as high) | Yes | Comparison of data, recorded on spread-sheets |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Correct sewage treatment work (STW) releases | Improved river flow conditions in vicinity of STW outflow. Specifics to be advised by Northumbria Water Limited (NWL) as part of PR14. | Fixed-point photography | Year round, but specifically before and after works (change in discharge from STW, bank management at confluence etc.). Several points | Volunteer on site | Historic observations and photos. Detailed report by site owner. | NWL / salary / volunteer | Hydrology: high | Priority: High | Yes | Comparison of photographs |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Increased interest of the Haltwhistle catchment and CRF project | Repeated activity and attendance at Halty CRF activities. | Sign in sheets, analyse who attends and who returns to other events. | After events and at the end of the year. | TRT | Some attendance lists of pre-project meetings exist | In-kind | Policies on public engagement | Priority: Medium | Yes | Comparison of numbers of people engaged and numbers of people returning for another event. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Increased understanding of how catchments work | Have the community raised their level of understanding of catchments. | Simple questionnaire | Early in project and at end of project. | TRT | None | In-kind | Policies on public engagement | Priority: Medium | Yes | Comparison of responses to questionnaire. Recorded and probably included in final report. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |