Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD objective is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver | Priority High/medium/low must link to priority of project targets ConfidenceHigh/medium/low robustness of monitoring | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use rrc update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Culvert removal | Increased fish passage | Mark re-capture with electrofishing. Fish are marked, some are then moved to downstream of the culvert, | Before and after ideally repeated up to three months after works completed | Salford Uni/ IRT/volunteers | none | Minimal; (in kind) | Ecology(HMWB) | Priority: H | yes | electrofishing is carried out to establish if any marked fish have moved up through where the culvert was. Need to mark sufficient fish such that re-capture is likely |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Fish easement (Larinier fishpass) | Increased fish passage | PIT tagging, Fish are tagged with a Passive Integrated Transponder (PIT). | Before and after ideally up to three immediately after installation after project end | Salford Uni/ IRT/volunteers | none | £2,000 | Ecology(HMWB) | Priority: H | yes | The tagged fish are traced using a receiver located in the fish pass to determine whether they are moving through the fish pass |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Weir removal | Increased fish passage and morphological processes | Fixed point photography | Before and immediately after ideally repeat the photos annually up to three years after project end | IRT | Yes | minimal | Ecology(HMWB) | Priority: H | yes | Comparisons of river before and after project. Continued photos up to three years would be useful |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Mark re-capture with electrofishing Fish are marked, some are then moved to downstream of the weir | Before and after ideally repeated up to three months after works completed | Salford Uni/ IRT/volunteers | none | Minimal; (in kind) |  | Priority: H | yes | electrofishing can then establish if any marked fish have moved up through where the weir was. Need to mark sufficient fish such that re-capture is likely |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Fluvial audit | Before and after, ideally up to three months after project and repeat up two years later | IRT | none | minimal |  | Priority: H | yes | Comparison of geomorphology before and after works. Ideally repeated 2 – 3 years after the project is completed |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| River blockage removal | Increased fish passage | Mark re-capture with electrofishing. Fish are marked, some are then moved to downstream of the blockage, | Before and after ideally repeated up to three months after works completed | Salford Uni/ IRT/volunteers | none | Minimal; (in kind) | Ecology(HMWB) | Priority: H | yes | electrofishing is carried out to establish if any marked fish have moved up through wher Need to mark sufficient fish such that re-capture is likely e the blockage was. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Fixed point photography | Before and immediately after ideally repeat the photos annually up to three years after project end | IRT | Yes | minimal |  | Priority: H | yes | Comparisons of river before and after project. Continued photos up to three years would be useful |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Litter collection | Reduction of litter along the river | Fixed point photography | Before and after ideally up to repeat 6 months after | IRT | No | minimal | Ecology(HMWB) | Priority: H | yes | Comparisons of amount of litter before and after collections. Education and signage regarding litter may help along with continued litter picking campaigns |
|  |  |  |  |  |  |  |  | Confidence H |  |  |

Reporting Note: There will be an end of project report and an end of project workshop