RRC Catchment Restoration Fund monitoring protocol

Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? |  | How What methods are you going to use? |  | When What periods over the year and how often? (to indicate variability) And where if possible |  | Who Who is going to do this? |  | Data What existing data is available in addition to the monitoring being outlined here |  | Cost (can be in kind) |  | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) |  | Priority High/medium/low linked to WFD or other designation |  | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. |  | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |  |  |
| Improvement in spawning habitat for fish and other aquatic species |  | Increase in fish abundance following habitat creation works (gravel augmentation at 15 sites; gravel loosening at 15 sites; and gravel washing at 10 sites) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator @ site, above (control sites) & one below each intervention site, in accordance with standard methods. Fixed point photography @ each site where physical works are completed (woody material, habitat improvement). |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Good baseline data along the Haddeo. Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to the CRF ones). |  | Through project with in-kind volunteer support (habitat works) |  | Haddeo (WB GB108045015090) Fish (Moderate) to (Good) |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post) |  |  |  |
| Improvement in instream cover and habitat for fish and other aquatic species |  | Increase in fish abundance following habitat creation works (selective coppicing and introduction of woody material (25 stems) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator @ site, above (control sites) & one below each intervention site, in accordance with standard methods. Fixed point photography @ each site where physical works are completed (woody material, habitat improvement). |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to CRF ones) – the new sample points upstream will be important in determining the outcome of these actions. |  | Through project with in-kind support from the local college |  | River Pulham (WB GB108045020910) Fish (Poor) to (Moderate) |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post) |  |  |  |
| Improvement in instream cover and habitat for fish and other aquatic species |  | Increase in fish abundance following habitat creation works (selective coppicing and introduction of woody material (50 stems) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator @ site, above (control sites) & one below each intervention site, in accordance with standard methods. Fixed point photography @ each site where physical works are completed (woody material, habitat improvement). |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to the CRF ones). |  | Through project |  | River Batherm (WB GB108045014860) Fish (Moderate) to (Good) |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post) |  |  |  |
| Reduction in intrusion of livestock and bank side erosion; & reduction in agricultural runoff through an improvement in farming practices |  | Mitigate bank erosion; and reduce/ mitigation substances which have an adverse impact on water quality and aquatic species prior to entering affected watercourses. |  | Fixed point photography @ each site where physical works are completed (fencing, restrictions on drinking areas for livestock). Farm demonstration workshops & soil tests will be made available to farmers throughout the catchment. |  | Fixed point photography pre and post habitat works Informal one-on-one conversations with 4/5 farmers in spring 2014 25 soil tests per year Diatom sampling @ farm & D/S where a water quality issue was identified |  | WRT staff Ben Goldsmith (UCL) – reporting on data sampled by WRT for diatoms |  | First data collected upstream of the ‘identified’ WFD sample point where failure was recorded in 2009 – so investigating source through this monitoring. |  | Through project |  | River Batherm (WB GB108045015070) Phytobenthos (Poor) to (Moderate) |  | Priority: High |  | Yes | Time-series of photographs Database of farm advice and grants, specifics what is granted. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (expected outcome within the timescale of the CRF programme) |  |  |  |
| Improve migratory fish passage for all species on the Exe, and improvement in instream cover and habitat for fish and other aquatic species |  | Change in fish population statistics (particular focus on salmon & trout) in habitats above 3 weirs (Pynes SX 917 626, Exwick SX 905 505 and Cowley SX 908 537) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator upstream and downstream of the weirs & @ habitat sites, undertaken in accordance with standard methods. Fixed point photography @ each site where physical works are completed (weir easement, selective coppicing and introduction of woody material). |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to the CRF ones). |  | Through project |  | River Exe (WB & GB108045015060) Fish (Moderate) to (Good) Improvements in water quality from upstream reaches should benefit Phytobenthos, currently failing (Moderate) in 3 Exe waterbodies |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (insufficient time within CRF to assess post weir works, but WRT has secured funds to assess success in 2015). |  |  |  |
| Improvement of river habitat on the River Culm & tributaries |  | Increase in fish abundance following habitat creation works (selective coppicing and introduction of woody material (25 stems) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator @ habitat sites, undertaken in accordance with standard methods (control sites depend on funding for the Sheldon Stream). Fixed point photography @ each project site where physical works are completed (woody material, habitat improvement). |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to the CRF ones). |  | Through project |  | Directly addresses: River Culm (WB GB108045015000) Fish (Poor) to (Moderate) Also: likely indirect improvement in Phytobenthos (Poor) to (Moderate); through CRF & Catchment Sensitive Farming programme. |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post) |  |  |  |
| Improve migratory fish passage for all species on the River Culm |  | Change in fish population statistics in habitats above baulk passes installed at two weirs (Whitehall and Smithincott) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator upstream and downstream of weirs, in accordance with standard methods. Fixed point photography @ each weir easement site. |  | All pre and post Electrofishing, summer 2012, 2013, 2014 Fixed point photography pre and post habitat creation |  | WRT staff (2012 baseline & 2014 survey) EA electrofishing did it summer 2013. |  | Existing data through other WRT projects & Environment Agency electro-fishing at sample sites in addition to the CRF ones). |  | Through project |  | River Culm (WB GB108045014980) Fish (Poor) to (Moderate) |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post) |  |  |  |
| Reduction in intrusion of livestock and bank side erosion; & reduction in agricultural runoff through an improvement in farming practices |  | Mitigate bank erosion and mitigate substances which have an adverse impact on water quality prior to entering failing watercourses. Diatom sampling to assess water quality failures |  | Fixed point photography @ each site where physical works are completed (fencing, installation of alternate drinking points for livestock). Farm demonstration workshops & soil tests Diatoms - Kelly et al. (2001) method. 5 stones scrubbed per sampling point |  | Fixed point photography pre and post habitat works 25 soil tests per year Diatom sampling (part of farm test) |  | WRT staff (FPP & soil testing) Ben Goldsmith (UCL) – reporting on data sampled by WRT for diatoms |  | Existing data collected through other WRT projects; Environment Agency WFD data (e.g. existing electro-fishing sample sites which complement the CRF sampling sites). |  | Through project |  | Blackwater (WB GB108045008850) Phosphate (Poor) to (Moderate) |  | Priority: High |  | Yes | Time-series of photographs Database of farm advice and grants, specifics what is granted. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (expected outcome within the timescale of the CRF programme) |  |  |  |
| Improve access to salmonids to the headwaters of the River Axe |  | Change in fish population statistics in habitats above 2 weirs (Clapton weir and at Manor Farm, Seaborough) |  | Semi-quantitative Fry index electro-fishing survey as fish abundance indicator upstream and downstream of weirs, in accordance with standard methods. Fixed point photography @ each weir easement site. |  | All pre and post Electrofishing, summer 2012 & 2014 Fixed point photography pre and post habitat creation |  | WRT staff |  | Existing data collected through other WRT projects; Environment Agency WFD data. |  | Through project |  | Axe (WBs GB108045014840 & GB108045008870) Also: likely indirect improvement in Phytobenthos (Poor) to (Moderate); through CRF & Catchment Sensitive Farming programme. |  | Priority: High |  | Yes | All electrofishing data (including EA data) uploaded to a GIS database. Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 1 post) |  |  |  |
| Reduction in direct run-off from forestry tracks to the river |  | Improvement in water and sediment quality after installation of sediment traps/ diverters |  | Redox potential within the salmon spawning sediments (gravels) within the river using pH samplers as a trial method. Fixed point photography @ each site where physical works are completed (woody material, habitat improvement) |  | All pre and post. Electrofishing, summer 2012, 2013, 2014 (Same site on Haddeo, stated earlier in table) Fixed point photography pre and post installation of sediment traps/ diverters Diatom sampling to assess water quality |  | WRT staff Plymouth University (redox pH trial) |  | Environment Agency have diatom data for lower Haddeo & flow data for catchment to assess flow curve longevity & influence of reservoir release |  | Through project |  | Trial as part of the CRF project May benefit Phytobenthos, which is currently failing (Moderate) in both Haddeo waterbodies but as trial method, unknown impact/s |  | Priority: Medium (investigative trial) |  | Yes | - pH - Redox potential - Diatom survey data Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post; trialled method) |  |  |  |
| Reduction in direct run-off from forestry tracks to the river |  | Improvement in water and sediment quality after installation of sediment traps/ diverters |  | Redox potential within the salmon spawning sediments (gravels) within the river using pH samplers as a trial method. Fixed point photography @ each site where physical works are completed (woody material, habitat improvement) |  | All pre and post Electrofishing, summer 2012, 2013, 2014 (Same site on Pulham, stated earlier in table) Fixed point photography pre and post installation of sediment traps/ diverters |  | WRT staff |  | No existing research/ data collection into sediment/ gravel. |  | Through project |  | Trial as part of the CRF project May benefit Phytobenthos in River Pulham (WB GB108045020910); but not stated as failing |  | Priority: Low (investigative trial, waterbody not failing) |  | Yes | - pH - Redox potential - Diatom survey data Time-series of photographs |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confidence: Medium (only 1 pre and 2 post; trialled method) |  |  |  |