Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD objective is this helping to achieve? If not WFD specify other e.g. CaBA | Priority High/medium/low linked to WFD or other designation Confidence High/medium/low robustness of monitoring | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use rrc update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wet Woodland Enhancement | Woodlands remain wetted | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Morphology, phosphorous & Diatoms | Priority: H | Yes, photos will be taken in spring before works are done ON TARGET | Comparisons of woodland habitat before and after works |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Woodland condition monitoring | Before and after (part of routine woodland survey protocol) | EXT | Previous surveys (6 years) |  |  | Priority: H |  | Comparison of changes in woodland condition before and after works |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued) |  |  |
| Ditch enhancement to wetlands | Prevention of choking up and drying out of wetland and ensure wetlands function in periods of heavy rainfall | Fixed point photography | Before works and up to 3 years after works | EWT | Will become part of routine site monitoring to ensure wetlands are functioning | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | Baseline photos and reports done , ditches dry in summer ON TARGET | Compare amounts of water retained within the ditches before and after the works are done (spring) |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Creation of Wetland Scrapes | Water retention during floods | Fixed Point photography | Before works and up to 3 years after works | EWT | no | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | This will be done next year ON TARGET | Compare amounts of water retained within the wet woodland before and after the works are done spring) |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Water levels | Wetlands will be monitored in spring until summer drawdown and this monitoring will continue into next year | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Measure levels in wetlands before and after in spring |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Installation of combination of coir rolls/ willow spiling and bank re-profiling | Reduction of erosion and sediment runoff into the watercourse | Fixed Point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Morphology, phosphorous & Diatoms | Priority: H | ON TARGET | Comparisons of emergent vegetation and flow variability before and after coir rolls put in |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | LEAFPAC macrophyte survey | Before works and at least three times after works, | EWT | yes | Part of EECOS (consultancy) contract Total value £7.2K |  | Priority: H | ON TARGET | Comparison of macrophytes assemblages before and after works at key downstream locations |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Macroinvertebrate (Riverfly) monitoring | Before works and up to 3 years after works | EWT/Volunteers | no | Total 2 days staff time = £500 + Volunteer input |  | Priority: M |  | Comparison of dominant macroinvertebrate assemblages before and after works. Plan is to continue monitoring after project has ended |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued |  |  |
| Stock Fencing | Reduced sedimentation | Fixed Point photography | Before works and up to 3 years after works | EWT | No | Total 2 days staff time = £500 Part of EECOS (consultancy) contract Total value £7.2K + Volunteer input | Phosphorous & Diatoms | Priority: H | ON TARGET | Comparison of overland flow and water turbidity before and after project |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | LEAFPAC macrophyte survey | Before works and at least three times after works, must be done for a similar rainfall event | EWT | yes |  |  | Priority: H |  | Comparison of macrophytes assemblages before and after works at key downstream locations |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Macroinvertebrate (Riverfly) monitoring | Before works and up to 3 years after works | EWT/Volunteers | no |  |  | Priority: M |  | Comparison of dominant macroinvertebrate assemblages before and after works. Plan is to continue monitoring after project has ended |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued |  |  |
| Reinstatement of back channels | Habitat improvement | Observe fish using back channel | Once before works and at least three times after works | EWT/Anglers/Landowners |  | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Compare use of backwater by fry before and after reinstatement |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Fixed Point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Compare marginal vegetation and habitats before and after reinstatement |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Introduction of wood debris | Increased flow variability | Fixed Point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Morphology | Priority: H | ON TARGET | Compare flow variability before and after introduction of LWD |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Create fish refuge areas | Observation/underwater camera | Once before works and at least three times after works | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Video or photographs would show if fish are using the large wood |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Fen restoration | Re-wetting of site by re-connecting to the river | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Comparisons of water levels after similar rainfall events at a similar time of year before and after restoration. |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Water levels (gauge board and pipewells) | Before works and up to 3 years after works | EWT |  |  |  | Priority: H |  | Monitor water levels before and after re-connection. Monotoring to continue after end of project |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Re-meandering | Habitat improvement | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Morphology, phosphorous & Diatoms | Priority: H | ON TARGET | Compare marginal vegetation and habitats before and after reinstatement |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Macroinvertebrate surveys (riverfly) | Before and after works. Needs to be in appropriate location and done up to 5 years after works, must be same time of year (spring or autumn) | Volunteers |  | Total 1days staff time = £250 + Volunteer input |  | Priority: M | ON TARGET | Needs to be done at same time of year (probably spring, summer and autumn |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued |  |  |
|  |  | Macrophyte surveys | Before and after works. Needs to be in appropriate location and done up to 5 years after works | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Comparison of macrophytes before (if any) to after reinstatement. Needs to be done at same time of year(Spring summer and autumn |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Hedgerow planting | Improve floodplain habitat | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Photograph or observation of the growth of hedgerows and the fauna using them. Could also observe runoff across the fields before the hedges are planted and compare with after to see if there is a reduction (would need to be a time of year - winter) |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Buffer Strips | Reduced sedimentation | Water sampling after storm events | Before works and at least three times after works, must be done for a similar rainfall event | EWT | Unknown | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Comparison of amount of sediment in the water column before and after works. Sample needs to be taken after comparable rainfall event and both downstream of the works and at a control site upstream |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Bank re-profiling | Morphological changes | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 | Morphology | Priority: H | ON TARGET | Compare bank and marginal vegetation before and after re-profiling (Same time of year) |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  |  | Macrophyte surveys | Before and after works. Needs to be in appropriate location and done up to 5 years after works | Volunteers/EWT |  |  |  | Priority: H |  |  |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Offline pond enhancement | Improvement in aquatic biodiversity | Pond dipping | Before works and at least three times after (needs to be same time of year) | EWT/Volunteers |  | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Comparison of variety of inverts before and after re-instatement |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued |  |  |
|  |  | Fixed point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Visual comparison of habitats and marginal and floating vegetation before and after re-instatement |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Ditch restoration | Wetting during storms | Fixed point photography | Before works and up to 3 years after works | EWT? |  | Total 2 days staff time = £500 | Phosphorous & Diatoms | Priority: H | ON TARGET | Visual comparison of amount of water retained in the ditches after similar rainfall event (and similar SMD) before and after restoration |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
|  | Silt retention | LEAFPAC macrophyte survey | Before works and at least three times after works, must be done for a similar rainfall event | EWT | yes | Part of EECOS (consultancy) contract Total value £7.2K |  | Priority: H | ON TARGET | Comparison of macrophytes assemblages before and after works at key downstream locations |
|  |  |  |  |  |  |  |  | Confidence H |  |  |
| Tree Thinning |  | Macroinvertebrate (Riverfly) surveys | Before and after works. Needs to be in appropriate location and done up to 5 years after works | EWT/volunteers |  | Total 1 days staff time = £500 + Volunteer time | Phosphorous & Diatoms | Priority: M | ON TARGET | Comparison of macroinvertebrates before to after thinning. Needs to be done at same time of year -spring/summer/autumn sampling to continue after project end |
|  |  |  |  |  |  |  |  | Confidence M (within CRF limit; High - if continued |  |  |
|  |  | Fixed Point photography | Before works and up to 3 years after works | EWT |  | Total 2 days staff time = £500 |  | Priority: H | ON TARGET | Visual comparison of bankside and marginal vegetation before and after thinning |
|  |  |  |  |  |  |  |  | Confidence H |  |  |

Reporting Note: Gains in biodiversity, improvements in water quality, community engagement and habitat restoration/creation will be qualified and reported in Quarterly Reports. Data collated will be submitted to the developing Local Records Centre (BRiE)