v“How to use” guide for the River Restoration Centre’s monitoring Protocol:

Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use RRC update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – for methodology, refer to RRC’s Practical river monitoring guidance (2011) X number of photos (state if known) & indicate if RRC have been provided with a map of points (Y/N) | E.g. Before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: Please state (only grey if High) | Yes/ No | A time-series of fixed point photographs State if any other analysis is being done |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if High) |  |  |

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) NOTE- can use rrc update questionnaires as a start. | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| To improve fish passage on the Wye catchment | Results of restoration of fish passage by removing barriers or making barriers passable | Electrofishing- 5 min riffle surveys to assess distribution across catchments WFD surveys at official monitoring points on waterbodies not within EA’s WFD fisheries monitoring programme. | Please see attached sheet. 58 5min surveys 7 sites taken to WFD standard Fished in late summer 2012,2013,2014 and using own funds in 2015. | Lugg and Arrow fisheries Association– landowner consents WUF: Planning, Conensting, delivering EA-some monitoring, consenting | Please ask EA for their data set. Location of fish passes Fixed point photography | £16,000 | Improve status of 23 waterbodies failing for fish- (improve moderate to good and poor to moderate) | Priority: High | The work at dayhouse weir has been delayed due to indecision by EA staff, but will happen this summer. The work on the other fish passes is proceeding as planned. The monitoring is on schedule | To be assessed in final project report to be carried out at end of project by project manager. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| To restore habitat for fish, invertebrates, diatoms and other species on the Wye and Usk | An improvement in habitat riparian and in channel (tree management, stock fencing to improve riparian buffer zone) | Fixed point photography | Before and after work | WUF staff | No | N/A | Improve status of waterbodies currently failing for invertebrates, diatoms and fish | Priority: High | On schedule | As above |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Redd counts | Annually subject to water conditions. In Lugg and Arrow | Volunteers | limited data going back to 1972 Annual data in arrow since 2006 | In Kind (£7,000 p/a) |  | Priority: High |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Invasive weed control- Log of invasive control activities and extent | Annually, to be recorded in a GIS | WUF staff and Volunteers | WUF annual survey since 2008 | £0 (funded through different project) |  | Priority: Medium |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
|  |  | Mapping on GIS | Uploaded in real time | WUF Staff | No | £6,213 |  | Priority: Medium |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| To reduce diffuse pollution, specifically phosphate and sediment by implementing targeted farm plans | A reduction in phosphate and sediment levels by stock fencing, soft revetment repair and farm infrastructure improvements | Diatoms 31 sites | 3 surveys p/a between 1st jan 2011 and 15th march 2015 (see attached file for network) | National Museum of wales | -EA water quality data for all WB’s. WB’s have diatom data. - In 2009 WUF assessed the Tippets brook for diatoms, Inverts and macrophytes | £59,220 | Improve status of waterbodies currently failing for phosphate | Priority: High | On schedule | As above |
|  |  |  |  |  |  |  |  | Confidence: Medium (diatoms surrogate for phosphate, but ideally would directly measure via water quality) |  |  |
|  |  | Fixed point photography | as and when work is completed |  |  | In-kind |  | Priority: High |  |  |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |