“How to use” guide for the River Restoration Centre’s monitoring Protocol:

Key:

- Target/why – What is the overall objective of the works which are to be monitored?

- What – What are you trying to observe from your monitoring? E.g. increased sinuosity and habitat heterogeneity through re-meandering and adding large wood / reduction in nutrient inputs by installing SuDS.

- How – What techniques are being used to collect data and what assessment methods are you using? E.g. electro-fishing monitoring diversity, abundance, density, length and age.

- When – When are you collecting data (month/season)? Duration/length of monitoring period, how many sampling repeats, how regularly?

- Who – Who is the individual and/or organisation responsible for monitoring? Will this be done by more than one organisation?

- Data – Do you have access to any pre-project data? E.g. monitoring data from the Environment Agency.

- Cost – Cost of monitoring. Are all costs in kind, or are there expenditures for e.g. external lab analysis.

- Which WFD objective is this helping to achieve – Which WFD quality element will be addressed by your works? If not WFD, does the work/undertaking aim to improve favourable conditions (for designated sites or species, e.g. SSSI/SAC/SPA/BAP) or does it relate to any other policy drivers (e.g. public engagement, socio-economics, flood management, ecosystem services)

- Priority and confidence: 
Priority: High/Medium/Low importance that your monitoring method can show potential improvement of the related WFD quality element; the favourable condition (i.e. designated site or species such as SSSI, SAC, SPA, BAP); and/or other policy drivers (e.g. socio-economics, flood management, ecosystem services).
Confidence: High/Medium/Low confidence that the monitoring is robust, suitable and has the potential to show what you are trying to observe within the CRF project time limit.

- On target – Are the monitoring tasks outlined running to schedule? If no, why not?

- Reporting tool and reporting output – How will your collected monitoring data be recorded and the analysis outputs reported?

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Will be different for each project – what is the project aim for the area being photographed? | A visual change in (please specify) as a result of (please specify) | Fixed point photography – (for methodology, refer to RRC’s Practical river monitoring guidance 2011) X number of photos (state if known) & if points indicated on map (Y/N) | E.g. before, immediately after and post works recommended (state dates if known, e.g. month and year) | Project team/ Volunteers | State if fixed point photography or any anecdotal/ ad-hoc photography prior to CRF | Through project/ In-kind | State which of the following, the FPP demonstrates: a) WFD targets, b) designated river or c) other e.g. social science targets | Priority: High (All CRF projects were encouraged to prioritise FPP) | Yes/ No | A time-series of fixed point photographs State if included in e.g. final report |
|  |  |  |  |  |  |  |  | Confidence: Please state (only grey if both confidence and priority are High) |  |  |

Example of Fixed Point Photography:

| Target/Why What is the overall objective of the works which are to be monitored? | What What are you trying to observe from your monitoring? | How What methods are you going to use? | When What periods over the year and how often? (to indicate variability) And where if possible | Who Who is going to do this? | Data What existing data is available in addition to the monitoring being outlined here | Cost (can be in kind) | Which WFD quality element is this helping to achieve? If not WFD specify (e.g. SSSI, SAC, BAP or other policy driver) | Priority High/medium/low linked to WFD or other designation | On target Are the monitoring tasks outlined running to schedule? (if no specify) | Key reporting tool and reporting output |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Confidence High/medium/low robustness of monitoring |  |  |
| Increase fish populations and improve instream habitat over approx. 1.5km of the River Windrush to contribute to achieve good ecological status by 2015. | Increased salmonid spawning habitats for winter 2012/13 spawning season and increased fish numbers from 2013 on by introducing high quality salmonid spawning gravel and large woody debris. | The fishing club is monitoring catch returns of <11” trout and grayling. | The club started counting catch returns 3-4y ago and will continue monitoring (not time bound). | Cotswold fly fishers | None | No cost | Fish: good Invertebrates: high Target: Good overall biological quality for 1.5km (failing for macrophytes) | Priority: Medium (as the WFD quality element, fish, already defined as good) | Yes | Compare data, recorded on form/spread sheets. Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit) |  |  |
|  |  | Electro-fishing survey (instead of redd counts) | Pre survey: 25th August 2013. Post survey: Planned in August 2016 Two sites on the Windrush, in restoration site and downstream. | EA |  | EA – no cost |  | Priority: Low (fish, already defined as good, post-monitoring after CRF time limit) | Yes | Compare data from before and after Data sent from EA |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit, but High if continued) |  |  |
|  |  | Invertebrates: monitored monthly by trained Riverfly person. 3min kick – standard Riverfly monitoring | Monitored monthly (except when flooding) since Dec 2011, monitoring is planned to continue (not time bound). | Cotswold Rivers Trust (CRT) | Monthly data since Dec 2011 | In kind |  | Priority: Medium (as the WFD quality element is already defined as high) | Yes | Completed standard forms are compared (same month each year) to establish changes in taxa/abundance |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit, but High if continued) |  |  |
| Improve the poor hydrological connectivity with the floodplain and consequential poor groundwater infiltration/ low degree of flood attenuation (R. Windrush). | Improved hydrological connectivity by removing elevated sections alongside the river, which were formed by the deposit of dredging from the river during 1930’s onwards. | Fixed-point photography (substitute for hydrological survey) Comparing location with previous photos (will record points on map) | No pre-photos available, but will continue with post monitoring. Quarterly | CRT | n/a | In kind | Hydrology: not high Targets: will look into setting targets | Priority: High | Yes | Comparing photos to see how the re-profiling and floodplain develops Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (no pre-photos, could be difficult to review hydrological condition from photos) |  |  |
| Increase water levels in the water meadow system and the number of breeding and over-wintering waders (R. Windrush). | Improved meadow habitats and reduced flood risk downstream from winter 2012/13 by installing simple flow control structures to enable controlled flooding. | Fixed-point photography (substitute for habitat monitoring) | Regularly (quarterly) | CRT | n/a | CRT (staff cost) | Hydrology: not high River/meadow has no other identified designation | Priority: High | Yes | Comparing photos to see how the habitats on the water meadows develops (wetland plant development etc) |
|  |  |  |  |  |  |  |  | Confidence: High (for reviewing meadow habitats, but Medium for hydrology) |  |  |
| Decrease diffuse agricultural pollution and Sedimentation (R. Windrush). | Decreased sediment input through fencing and provision of alternative water supplies. | Invertebrates – 3min kick. Will look into using PSI/BMWP indices | Monitored monthly, planned to continue (not time bound) | CRT | Monthly data since Dec 2011 | CRT (staff cost) | Invertebrates: high Overall physic-chemical quality: high Overall specific pollutants quality: high Morphology: good | Priority: Medium (as inverts and water quality already defined as high) | Yes | Comparing samples to see if pollution /sediment sensitive species increase in taxa and abundance. Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit, but High if continued) |  |  |
|  |  | Fixed-point photography (substitute for sediment survey) | Quarterly |  | n/a |  |  | Priority: High |  | Comparing photos to see how banks/veg develop after fencing, and if gravel keeps clear of silt. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Changes to track drainage, cattle drinking areas, and the reduction of access to the Sherborne Brook by livestock. | Decreased sediment input through fencing and alternative water supplies. Creation of coarse vegetation and deciduous tree buffer strips. | Invertebrates – 3min kick. Will look into using PSI index | Monthly monitoring commenced Sept 2013 (pre-works), planned to continue (not time bound) | CRT | none | CRT (staff cost) | Morphology: good | Priority: Medium (as morphology already defined as high) | Yes | Comparing samples to see if sediment sensitive species increase in taxa and abundance. Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit, but High if continued) |  |  |
|  |  | Fixed-point photography (substitute for sediment survey) | Quarterly |  | n/a |  |  | Priority: High |  | Comparing photos to see how banks/veg develop after fencing, and if gravel keeps clear of silt. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |
| Reduction of over-shading on the Sherborne Brook. | Reduction of over-shading by selective coppicing on approximately 800m section of the brook. | Macrophytes - light, medium, heavy cover | Will start monitoring now – spring/summer | CRT | none | CRT (staff cost) | Macrophytes: moderate Targets: will look into setting targets | Priority: High | Monitoring not started | Anecdotal and looking at development of macrophytes after coppicing. Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (as no pre-data exists) |  |  |
| Improve instream habitats by introduction of large woody debris. | Improved instream habitats by introduction of large woody debris. | Invertebrates - 3min kick sampling and standard Riverfly monitoring. | Monthly monitoring commenced Sept 2013 (pre-works), planned to continue (not time bound) | CRT | none | CRT (staff cost) | Invertebrates: high Morphology: good Hydrology: not high | Priority: Medium (as inverts already defined as high) | Yes | Completed forms are compared (same month each year) to establish changes in taxa/abundance. Final report planned (but not until 4-5 y of post monitoring) |
|  |  |  |  |  |  |  |  | Confidence: Medium (within CRF time limit, but High if continued) |  |  |
|  |  | Fixed-point photography (substitute for habitat survey) | Quarterly |  | n/a |  |  | Priority: High |  | Comparing photos to see how created habitats succeed and develops. |
|  |  |  |  |  |  |  |  | Confidence: High |  |  |